ID: 24101203  CSE 421

Application Layer Protocols (HTTP.SMTP/POP)

Examination Lab

Objectives:

Capture traffic and observe the PDUS for HTTP, SMTP, POP.

Task 1: Observe HTTP traffic exchange between a client and server.  Step 1 – Run the simulation and capture the traffic.

• Enter Simulation mode.

• Click on the PC1. Open the Web Browser from the Desktop.

• Enter www.bracu.ac.bd into the browser. Clicking on Go will initiate a web server  request. Minimize the Web Client configuration window.

• Two packets appear in the Event List, a DNS request needed to resolve the URL to  the IP address of the web server and an ARP request needed to resolve the IP  address of the server to its hardware MAC address.

• Click the Auto Capture / Play button to run the simulation and capture events.  • Sit tight and observe the packets flowing through the network.

• When the above message appears Click “View Previous Events".

• Click on PC1. The web browser displays a web page appears.

Step 2 – Examine the following captured traffic.

Our objective in this lab is only to observe HTTP traffic.

|  | Last Device | At Device | Type |
| --- | --- | --- | --- |
| 1. | PC1 | Switch 0 | HTTP |
| 2.. | Local Web Server | Switch 1 | HTTP |

• Find the following packets given in the table above in the Event List, and click on the  colored square in the Info column.

• When you click on the Info square for a packet in the event list the PDU  Information window opens. If you click on these layers, the algorithm used by the device  (in this case, the PC) is displayed. View what is going on at each layer.

• Examine the PDU information for the remaining events in the exchange. For packet 1::

What kind of HTTP packet is packet no. 1?

Packet for the pc1 is a request HTTP packet to retrieve the web server

Click onto “Inbound PDU details” tab. Scroll down at the end, what do you see?

At the bottom of the tab it shows the HTTP header and the accepted language and field

For packet 2:

Click onto the “Inbound PDU details” tab. Scroll down at the end, what do you see? What kind of  HTTP packet is this?

This Packet is a HTTP response packet containing the information regarding web servers source port =80, destination port = 1040 and header for the HTTP showing the connection : close and content length as 151

Task 2: Observe email traffic exchange between a client and email  server using SMTP and POP3.

Step 1 – Run the simulation and capture the traffic.

• On the Event List window click “Reset Simulation” button. All previous packets will  disappear.

• At the bottom of the Event List window, there is a filter which filters the protocols that  we want to see. Click Edit filters. Another window appears showing different  protocols, unclick HTTP and click SMTP and POP3.

• Click a space anywhere outside the popup window, then it will disappear.  • Your Event List Filter should be as shown below:

2

• Now click on the PC1. Close the web browser window. Open the Email from  the Desktop. A mail browser window will open. Click “compose”, another window  appears.

• Fill the window as shown and press send.

• Minimize the client window .

• Click the Auto Capture / Play button to run the simulation and capture events.  • Sit tight and observe the packets flowing through the network.

• This interaction is between the sender client and its email server.

Step 2 – Examine the following captured traffic.

Our objective in this lab is only to observe SMTP traffic.

|  | Last Device | At Device | Type |
| --- | --- | --- | --- |
| 3. | PC1 | Switch 0 | DNS |
| 4. | PC1 | Switch 0 | SMTP |
| 5. | Bracu Email Server | Switch 1 | SMTP |

• Find the following packets given in the table above in the Event List, and click on the  colored square in the Info column.

• Examine the PDU information.

For packet 4::

What is the purpose of this DNS packet?

The DNS packet is converting the domain name to an IP address source:(192.168.10.1 ), destination: 92.168.10.2  , which allows the communication between the source and destination.

For packet 5& 6::

Explain why SMTP packet was sent to the email server and the server replied with an SMTP  packet?

The SMTP is the protocol for mail transfer and an SMTP packet was sent from pc1 to the mail server for delivering the mail and the server also replied with an SMTP packet to correspond the request and confirm the command. So that the email transmission to continue

3

Step 3 – Run the simulation and capture the traffic for POP.

• On the Event List window click “Reset Simulation” button. All previous packets will  disappear.

• Now click on the PC0. Open the Email from the Desktop. A mail browser window will  open. Click “receive”, minimize the window.

• Click the Auto Capture / Play button to run the simulation and capture events.  • Sit tight and observe the packets flowing through the network.

• This interaction is between the sender client and its email server.

Step 2 – Examine the following captured traffic.

Our objective in this lab is only to observe POP traffic.

|  | Last Device | At Device | Type |
| --- | --- | --- | --- |
| 6. | PC0 | Switch 0 | DNS |
| 7. | PC0 | Switch 0 | POP3 |
| 8. | Bracu Email Server | Switch 1 | POP3 |

• Find the following packets given in the table above in the Event List, and click on the  colored square in the Info column.

• Examine the PDU information.

For packet 6::

What is the purpose of this DNS packet?

The DNS is again converting the domain name to its corresponding IP address to retrieve the mail and connect the server to the client(pc0) source IP:192.168.10.10 and the destination IP:192.168.10.2. The protocol is UDP

For packet 7&8::

Explain why the POP packet was sent to the email server and the server replied with a POP  packet?

The packet 7 of POP3 is needed to connect the POP3,  to start retrieving mail and the packet 8 is the reply from the server acknowledging the connection. This is because of the TCP connection between the client and the server.Here source port is 1025 and dest port is 110

•

4